system Overview

The system has two main computers talking to each other over WiFi:

Raspberry Pi - lives on the robot. Runs Flask on port 5000. Controls motors, reads sensors via Arduino, streams video, and saves data to SQLite.

PC (control station) - runs a second Flask server on port 8000 that acts as a proxy, forwarding all requests to the Pi and serving the web interface to your browser.

The reason for this two-server design is that your browser connects to localhost (the PC), which keeps things simple and avoids CORS issues with direct Pi access from a browser.

On the hardware side there are three main layers:

Movement: two DC motors driven by an L298N dual H-bridge, controlled by the Pi's GPIO pins

Sensing: an Arduino reads ultrasonic distance sensors and sends wall-detection messages to the Pi over serial (USB)

Vision: an OV5647 camera on the Pi streams live video and captures images at high-moisture location

Software and Dependencies

Arduino Libraries (install via Arduino IDE Library Manager):

Arduino_LSM6DS3: for the built-in IMU/gyroscope on the Nano 33 IoT

TinyGPSPlus: for parsing NMEA sentences from the GPS module

To install these, open the Arduino IDE, go to Sketch -> Include Library -> Manage Libraries, then search for each one by name and hit install.

On the Raspberry Pi:

pip install flask flask-cors pyserial opencv-python

Also install picamera2 if using the Pi camera module properly:

sudo apt install python3-picamera2

On the PC:

pip install flask flask-cors requests

Project Structure

Before seeing the code this is how the project is laid out:

project/

│

├── main.py              # Entry point  boots everything on the Pi

├── config.py            # All hardware pins and constants in one place

├── state.py             # Shared robot state (mode, running, etc.)

├── saveData.py          # SQLite + ring buffer data manager

├── frontend.py          # PC-side proxy server

│

├── arduino/

│   ├── Main.ino         # Entry point, GPS and main loop

│   ├── ultrasonic.ino   # Wall avoidance and IMU logic

│   ├── moisture.ino     # Moisture sensor and interrupt

│   ├── ultrasonic.h     # Shared ultrasonic variables

│   └── moisture.h       # Shared moisture variables

│

├── hardware/

│   ├── motors.py        # L298N motor controller

│   ├── camera.py        # Camera manager (picamera2/picamera/OpenCV)

│   └── arduino.py       # Serial connection and message parser

│

└── api/

├── app.py           # Flask app factory

├── autonomous.py    # Autonomous driving and sensor loop

└── routes/

├── control.py   # Motor command endpoints

├── data.py      # Readings and stats endpoints

└── stream.py    # Video stream endpoint

This separation is intentional - each file has one job. If you need to change a motor pin, you only change

config.py

If you want to change how data is saved, you only touch

saveData.py

this keeps the the code clean and easy to maintain

Step 1: Configuration (config.py)

The first and most important principle of this project is that every hardware constant lives in one file. Never hardcode a pin number or threshold inside a function.

python

# Motor GPIO pins (BCM numbering)

MOTOR1_IN1 = 17

MOTOR1_IN2 = 27

MOTOR1_PWM = 22

MOTOR2_IN1 = 23

MOTOR2_IN2 = 24

MOTOR2_PWM = 25

MOTOR_DEFAULT_SPEED = 60  # %

# Moisture thresholds

MOISTURE_CRITICAL = 80

MOISTURE_MODERATE = 60

# Arduino serial

SERIAL_PORT = '/dev/ttyACM0'  # Change to 'COM6' on Windows

SERIAL_BAUD = 9600

You notice the IS_PI flag at the top of config.py - it detects whether the code is running on a Pi or a regular PC using platform.machine(). This is used throughout the project to swap in mock hardware on your PC so you can develop and test without physical hardware present.

this is useful for things like testing the future API and server changes before having them run on the pi directly

Step 2: Shared State (state.py)

Rather than passing configuration dictionaries between threads, a single shared state dict is imported wherever it's needed:

python

state = {

'mode':      'autonomous',  # 'autonomous' | 'manual'

'running':   True,

'streaming': False,

'start_time': time.time(),

}

When the operator switches to manual mode from the web interface, the autonomous loop reads state['mode'] and pauses itself. This avoids race conditions from multiple threads trying to drive the motors simultaneously.

Step 3: Motor Controller (hardware/motors.py)

The L298N motor driver takes two digital direction pins and one PWM speed pin per motor. The _set_motor method handles all three cases forward, reverse, and stop. by setting the direction pins and adjusting PWM duty cycle:

python

def _set_motor(self, in1, in2, pwm, speed):

speed = max(-self.max_speed, min(self.max_speed, speed))

if speed > 0:

GPIO.output(in1, GPIO.HIGH)

GPIO.output(in2, GPIO.LOW)

pwm.ChangeDutyCycle(speed)

elif speed < 0:

GPIO.output(in1, GPIO.LOW)

GPIO.output(in2, GPIO.HIGH)

pwm.ChangeDutyCycle(-speed)

else:

GPIO.output(in1, GPIO.LOW)

GPIO.output(in2, GPIO.LOW)

pwm.ChangeDutyCycle(0)

the MockGPIO class is for when IS_PI is False, a fake GPIO class is substituted that simply prints what it would do instead of touching real hardware. This is for testing the motor logic on your PC first.

For turning, rather than stopping and pivoting on the spot, the robot uses differential drive. one motor slows down relative to the other based on the angle received from the Arduino:

python

def turn_left_angle(self, angle):

reduction = min(abs(angle) / 90.0 * self.max_speed, self.max_speed)

self._set_motor(MOTOR1_IN1, MOTOR1_IN2, self.pwm1, self.current_speed)

self._set_motor(MOTOR2_IN1, MOTOR2_IN2, self.pwm2,

max(self.current_speed - reduction, 0))

Step 4: Arduino Hardware Overview

The Arduino (an Arduino Nano 33 IoT is used here for its built-in IMU but any Arduino could work as long as it is compatible with the hardware) is the sensor hub of the robot. It handles three things independently and reports everything to the Pi over USB serial at 9600 baud:

Wall/obstacle detection: two HC-SR04 ultrasonic sensors measure distances on either side of the robot

Moisture sensing: an analog moisture sensor on pin A0, with a hardware interrupt on pin 2 for threshold alerts

GPS: a GPS module connected to Serial1 at 115200 baud, parsed using the TinyGPSPlus library

The Arduino sketch is split across multiple .ino files that the Arduino IDE automatically combines. Main.ino is the entry point, with ultrasonic.ino and moisture.ino containing the hardware logic, and header files defining shared variables.

Step 4a: Serial Protocol

Every message the Arduino sends follows a consistent prefix format so the Pi can parse them:

READY                          sent once on boot

SAFE                           both ultrasonic sensors clear

ULTRASONIC: WALL:d1,d2,angle   obstacle detected

MOISTURE:raw,DRY|MOIST|WET     regular moisture reading

MOISTURE:THRESHOLD_TRIGGERED   interrupt fired (sudden wet event)

GPS:lat,lng                    valid GPS fix

GPS:NO_FIX                     waiting for satellite lock

This design choice means the Pi only needs to check the prefix of each line to read what kind of data arrived.

Step 4b: Moisture Sensor with Hardware Interrupt

The moisture sensor is read every second via readMoisture(). Raw analog values are mapped to three states using tunable thresholds:

const int DRY_THRESHOLD = 600;  // above this = dry

const int WET_THRESHOLD = 300;  // below this = wet

void readMoisture() {

moistureValue = analogRead(MOISTURE_PIN);

Serial.print("MOISTURE:");

Serial.print(moistureValue);

Serial.print(",");

if      (moistureValue >= DRY_THRESHOLD) Serial.println("DRY");

else if (moistureValue <= WET_THRESHOLD)  Serial.println("WET");

else                                      Serial.println("MOIST");

if (thresholdExceeded) {

thresholdExceeded = false;

Serial.println("MOISTURE:THRESHOLD_TRIGGERED");

}

}

A hardware interrupt on pin 2 watches for a sudden falling edge (a rapid wet event) separately from the polling loop. It uses a 200ms debounce to avoid false triggers:

void moistureThresholdISR() {

unsigned long now = millis();

if (now - lastInterruptTime > DEBOUNCE_DELAY) {

thresholdExceeded = true;

lastInterruptTime = now;

}

}

This means the robot gets both regular readings every second AND an immediate alert if moisture spikes suddenly. this is useful for detecting a leaking pipe versus general dampness.

Step 4c: Dual Ultrasonic Wall Avoidance with IMU

Two HC-SR04 sensors are mounted at the front of the robot, separated by a known distance L = 11.5cm. By comparing both distances, the robot can calculate the angle it is approaching a wall at, not just whether something is in front of it.

A key point for two ultrasonic sensors is crosstalk: sensor 1's sound waves can be picked up by sensor 2's echo pin. The solution is a 30ms delay between firing them:

void updateWallAvoidance() {

float d1 = getDistance(TRIG_PIN1, ECHO_PIN1);

delay(30);  // let sensor 1's waves die out completely

float d2 = getDistance(TRIG_PIN2, ECHO_PIN2);

...

}

The wall angle is calculated using trigonometry. the atan of the distance difference divided by the sensor separation:

wallAngle = atan((d2 - d1) / L) * (180.0 / 3.14159265);

The turn angle is then calculated dynamically based on how close the obstacle is. the closer the wall, the sharper the turn, scaled between 15° and 45°:

float determineTurnAngle(float dist1, float dist2) {

float base = ((SAFE_DISTANCE - minDistance) / SAFE_DISTANCE)

* (MAX_TURN_ANGLE - 15.0) + 15.0;

base = constrain(base, 15.0, MAX_TURN_ANGLE);

if      (dist1 < dist2) return -base;  // obstacle on left, turn right

else if (dist2 < dist1) return  base;  // obstacle on right, turn left

else return (currentGyroAngle < 0) ? -base : base;

}

The built-in IMU (LSM6DS3 gyroscope) tracks the robot's current heading so the Pi can confirm how much the robot has actually turned, not just how much it was told to turn.

photo:

Step 4d: GPS

The GPS module is connected to Serial1 on the Arduino at 115200 baud (this might differ dependent on what and where you get you gps). The TinyGPSPlus library handles NMEA sentence parsing. In the main loop, GPS data is fed into the parser continuously and reported to the Pi once per second alongside the moisture reading:

void feedGPS() {

while (Serial1.available()) gps.encode(Serial1.read());

}

void sendGPS() {

if (gps.location.isValid()) {

Serial.print("GPS:");

Serial.print(gps.location.lat(), 6);

Serial.print(",");

Serial.println(gps.location.lng(), 6);

} else {

Serial.println("GPS:NO_FIX");

}

}

(Note. GPS:NO_FIX is sent until the module acquires satellite lock, which can take 30–60 seconds outdoors on first boot. The Pi-side health monitor (health.py) checks for this and marks CAN_GPS as false until a real fix arrives.)

Step 4e: Main Loop Timing

One important detail in Main.ino is how the loop is structured to avoid blocking. pulseIn() for ultrasonic sensors can block for up to 30ms waiting for an echo. which would starve the GPS parser of bytes and cause missed NMEA sentences:

void loop() {

unsigned long now = millis();

feedGPS();               // always runs first, never blocked

if (now - lastSonicTime >= sonicInterval) {  // every 100ms

lastSonicTime = now;

updateWallAvoidance();

}

if (now - lastReading >= readingInterval) {  // every 1000ms

lastReading = now;

readMoisture();

sendGPS();

}

}

GPS feeding runs every iteration, while ultrasonic checks are rate-limited to every 100ms and moisture/GPS reporting happens every 1000ms. This keeps all three systems responsive without any one blocking the others.

photo:

Step 5: Arduino/Pi Communication (hardware/arduino.py)

The Arduino reads ultrasonic sensors and sends one of two message types over serial USB to the Pi:

SAFE: path is clear, keep going

WALL:remaining,angle,amount: obstacle detected with navigation data

The Pi side parses these with a simple function:

python

def parse(line):

if line == 'SAFE':

return {'type': 'safe'}

if line.startswith('WALL:'):

parts = line[5:].split(',')

return {

'type':      'wall',

'remaining': float(parts[0]),

'angle':     float(parts[1]),

'amount':    float(parts[2]),

}

return None

If the Arduino fails to connect on startup (wrong port, not plugged in, etc.), the code catches the exception and sets arduino = None. Every place that uses the Arduino checks for None first, so the rest of the robot keeps working even without wall avoidance.

Step 6: Camera (hardware/camera.py)

The camera manager tries three different backends in order, falling back gracefully:

picamera2: the modern Pi camera library (preferred)

picamera: the older Pi camera library

OpenCV VideoCapture: works on any device including your PC webcam

This fallback function means the same code runs on a Pi with a working camera module, a Pi with a USB webcam, or your laptop during development or hardware changes.

Streaming works by running a background thread that captures frames at the configured FPS, JPEG-encodes them, and stores the latest one in a thread-safe buffer. The Flask route then reads from this buffer and serves it as a multipart HTTP stream.

When moisture exceeds the moderate threshold, capture_image() is called. It overlays the timestamp, moisture reading, and GPS coordinates directly onto the frame before saving it, so each image is self-documenting.

Step 7: Data Storage (saveData.py)

Data is stored in two layers simultaneously:

SQLite database: persistent across reboots, written in a background thread so it never blocks the main loop

In-memory ring buffer: a deque with a max size of 500 readings for fast API responses

On startup, the ring buffer is pre-filled ("warmed") from the database so the dashboard immediately shows historical data.

python

def add_moisture_reading(self, gps_lat, gps_lng, moisture, ...):

# Write to ring buffer immediately (fast, for API reads)

with self._lock:

self._buffer.append(row)

# Write to SQLite in a background thread (slow, but persistent)

threading.Thread(target=self._write_db, ..., daemon=True).start()

Each reading stores GPS coordinates, moisture percentage, IR temperature, severity classification, and an optional image filename.

Step 8: The Autonomous Loop (api/autonomous.py)

This is the brain of the robot. It runs in a background thread and does two things every iteration:

Check for wall messages from the Arduino and steer dependent on its message

Every 5 seconds, read sensors, classify severity, optionally capture an image, and save the reading

python

while state['running']:

if state['mode'] != 'autonomous':

time.sleep(0.1)

continue

# Wall avoidance

line = read_line()

parsed = parse(line)

if parsed:

if parsed['type'] == 'wall':

rem = parsed['remaining']

if rem > 5:

motors.turn_left_angle(rem)

else:

motors.forward()

elif parsed['type'] == 'safe':

motors.forward()

# Sensor save (every SENSOR_SAVE_INTERVAL seconds)

if now - last_save >= SENSOR_SAVE_INTERVAL:

sensor = read_sensors()

severity = _determine_severity(sensor['moisture'])

storage.add_moisture_reading(...)

if severity == 'Critical':

motors.stop()

time.sleep(2)  # Pause for inspection

When a Critical reading is detected the robot stops for 2 seconds this gives the system time to capture a clear image before moving on.

Step 9: The Web Interface

The frontend consists of four pages served by Flask:

Dashboard (index.html): shows live stats (total readings, average moisture, critical count) and a table of recent readings with severity badges. Auto-refreshes every 10 seconds.

Control (control.html): live video feed on the left, movement controls on the right. Supports on-screen buttons and keyboard (arrow keys to move, space bar to stop). Shows current mode (Manual/Autonomous) with a color badge.

Map (map.html): displays current GPS position and is structured to integrate a mapping library.  // this is depreciated and doesnt have any current functionalty

Report (report.html): summary statistics, system health table, and export options. // this is depreciated and doesnt have any current functionalty

The control page's mode switching is worth noting. when you click Autonomous, the movement buttons are still visible but commands sent while in autonomous mode are rejected by the Pi and a message is shown instead. The robot stays in control of itself unless you hit stop which will prevent the robot from moving in any situation.

Step 10: The PC Proxy (frontend.py)

Rather than having your browser talk directly to the Pi's IP address, all traffic goes through a second Flask server running on your PC:

Browser -> localhost:8000 (frontend.py) -> 10.x.x.x:5000 (Pi)

This proxy caches the connection status for 5 seconds to avoid flooding the Pi with health checks, handles timeouts gracefully, and proxies the video stream using chunked transfer. To point it at your Pi, change one line:

python

PI_SERVER = 'http://YOUR_PI_IP:5000'

Step 11: Running the Project

Finding the Pi's IP address

Before you can run the frontend, you need to know the Pi's current IP address. This is important: the Pi's IP address can change every time you connect it to a different network or hotspot. It is assigned dynamically by the router (DHCP), so if you switch from your home WiFi to a phone hotspot, or connect to a different router at university, the address will be different.

To find the current IP, either connect a monitor and keyboard to the Pi and run:

bash

hostname -I

Or if you can SSH into it:

bash

ssh pi@raspberrypi.local

hostname -I

The output will look something like ##.###.###.### copy that address.

Updating the frontend

Once you have the IP, open frontend.py on your PC and update this one line before running:

python

PI_SERVER = 'http://YOUR_PI_IP:5000'

You need to do this every time the Pi connects to a different network. A good habit is to check hostname -I on the Pi first thing whenever you move to a new location. ( if you have an android you can configure your hotspot setting to never change the IP address for consistency)

Running everything

On the Raspberry Pi:

bash

cd project

python3 main.py

On your PC:

bash

cd project

python3 frontend.py

Then open http://localhost:8000 in your browser. The dashboard will show a red "Pi Not Connected" indicator if the IP in frontend.py is wrong or stale.

To test motors before a full run:

bash

python3 motor_test.py

This project was submitted to Deakin University, School of IT, Unit SIT210/730 - Embedded Systems Development