# Water Damage Detection Robot Prototype Report

SIT210 – Embedded Systems Development

Student Name: Charlie Mason

Student ID: 223391013

Date: 29/04/2026

Table of Contents

1. Project Overview	3

1.1 Background	3

1.2 Existing Work	4

1.3 Problem Statement	4

1.4 Requirements	5

2. Prototype Architecture	6

2.1 Hardware Architecture	6

2.2 System Block Diagram	7

2.3 Network Architecture & Flow Diagram	8

3. Implementation & Source Code	9

3.1 Repository Link	9

3.2 Working Mechanism	9

4. Evaluation & Testing	9

4.1 Testing Approach	9

4.2 Evaluation Criteria	11

5. Advanced Design & Fault Tolerance (D/HD Requirements)	12

5.1 Fault Tolerance Strategy	12

5.2 Design Principles (HD Requirement)	13

6. User Manual	14

6.1 Installation Guide	14

6.2 Operating Instructions	15

7. Project Demonstration	16

8. Conclusion & Reflection	16

8.1 Summary	16

8.2 Reflection	16

9. Bibliography	17

## 1. Project Overview

### 1.1 Background

Residential homes in Australia are increasingly vulnerable to water damage originating from beneath the structure. The crawlspaces under peoples’ homes are often neglected and are a common place for moisture to accumulate due to leaks, condensation and ground water. Because these crawlspaces are small, poorly lit, and difficult for humans to access. This leaves moisture problems to frequently go undetected until they have caused significant structural damage, mould growth, or degradation of flooring and framing timbers.

In Australia, internal water damage is now a more costly risk to a household than fire and burglary, with average water damage claims rising by 72% over five years according to CHUBB. Data from Proximity Plumbing 24% of all Australian home insurance claims relate to water damage, with an average claim of around $5,000 and severe losses exceeding $500,000.

Current inspection requires a tradesperson or building inspector to physically crawl through the subfloor space, which poses occupational health and safety risks and is limited by human endurance, lighting, and the physical dimensions of the space.

This project proposes a semi-autonomous robotic platform capable of navigating the subfloor environment and automatically detecting and geo-tagging points of excess moisture using camera and moisture detection sensors, while retaining the option for human control. the data is saved and accessible from the robot and can be viewed from a local dashboard.

### 1.2 Existing Work

Several robotic platforms exist for confined-space and environmental inspection, though none specifically address autonomous geo-referenced moisture mapping in residential subfloors.

Consumer floor robots such as the iRobot Roomba navigate flat indoor surfaces using infrared and cliff sensors, but their sensor suite is limited to obstacle detection and floor cleaning.  they carry no environmental sensing payload and are not designed for subfloor clearances. In the academic space, research into autonomous confined-space navigation has been researched considerably. 2022 a survey published in Robotics and Autonomous Systems (ScienceDirect) identifies robust map generation, precise localisation, and safe navigation as known challenges for confined and subterranean robotic system.

A Tennessee Tech CE capstone project developed an autonomous crawlspace inspection robot with temperature, humidity, and moisture sensing, achieving 96% environmental sensing accuracy. This shows us the feasibility but remains a non-commercial research prototype.

No current solution combines autonomous navigation, geo-referenced moisture logging, and remote human override in a single low-profile platform specifically designed for subfloor deployment.

### 1.3 Problem Statement

Undetected moisture is a leading cause of property damage and degradation from timber rot, mould and other hazardous results. Despite its prevalence and cost early detection is often quite primitive requiring manual inspection inside tight and in accessible crawlspaces. Current existing robotic solutions require human operation and don’t produce a geo reference record of moisture and its change over time across the subfloor area.

This prototype aims to address this gap by being a semi-autonomous embedded robot that can navigate the subfloor environment and with minimal human intervention to systematically map the moisture levels using a combination of geo tracking and moisture sensor technology.

### 1.4 Requirements

functional requirements:

- The robot shall detect moisture levels via a resistive moisture sensor and classify readings as Minor, Moderate, or Critical based on configurable thresholds.

- The robot shall record the GPS coordinates of each moisture reading and store them persistently in a SQLite database.

- The robot shall capture a JPEG image annotated with moisture level, GPS coordinates, and timestamp whenever a Moderate or Critical reading is detected.

- The robot shall navigate autonomously using ultrasonic sensors to detect and avoid walls and obstacles, stopping and turning when an object is within 15 cm.

- The robot shall support manual override via a web-based control panel, accepting forward, backward, left, right, and stop commands with adjustable speed.

- The system shall stream live video from the onboard camera to the operator's PC in real time.

- The frontend shall display a GPS map of all readings, colour-coded by severity.

- The system shall switch between manual and autonomous modes on operator command.

Non-functional requirements:

- The Pi-side Flask server shall respond to control commands within 2 seconds under normal WiFi conditions.

- The system shall tolerate the loss of any single sensor (GPS, ultrasonic, camera) without crashing; remaining subsystems shall continue to operate.

- Sensor data shall persist across power cycles via SQLite.

- The in-memory ring buffer shall retain the 500 most recent readings for low-latency frontend access.

- All motor GPIO resources shall be cleanly released on shutdown or SIGINT/SIGTERM.

## 2. Prototype Architecture

### 2.1 Hardware Architecture

Motors x2: connected to the motor controller for power

L892N motor controller: connected to the Raspberry Pi and a battery pack. Controls the motors direction and power output.

Camera module:  connected to the pi for image and video streaming

Raspberry pi 4: connected to the motor controller to regulate the direction and power output, connected to the Arduino via serial to receive sensor info and wall detection. Connected to the camera to take video and pictures of detected moisture.

Arduino nano iot33: connected to the raspberry pi via serial. Connected to gps, ultrasonic sensor and moisture sensor.

Gps Module: finds the robot position to help map locations of detected moisture.

Moisture sensor:  detects moisture on the ground

Ultrasonic sensor x2: detects walls and obstacles. Will help calculate the direction to turn by finding the angle of the obstruction

Battery pack: x2: powers everything

### 2.2 System Block Diagram

The system is built in three physical tiers. first tier is the Arduino Nano 33 IoT which interfaces directly with the sensors: a resistive moisture sensor on an analogue pin, two HC-SR04 ultrasonic sensors via interrupt-driven echo capture, and a GPS module via UART. The Arduino processes raw readings, applies thresholds, and emits structured serial messages (e.g. MOISTURE:327,MOIST, ULTRASONIC:WALL:12.3,14.1,5.2, GPS:lat,lng) at 9600 baud

In the middle tier, the Raspberry Pi 4 reads those serial messages, parses them in hardware/arduino.py, and maintains a shared in-memory sensor state updated on every loop iteration in api/autonomous.py. The Pi drives two DC motors via an L298N H-bridge, hosts a Flask REST API on port 5000, manages the camera stream, and writes readings to SQLite via saveData.py.

At the last tier, the operators PC runs a second Flask server (frontend.py) on port 8000 that proxies all requests to the Pi over WiFi. The browser loads control.html, map.html, and report.html which call the proxy API to send commands and display live data and images.

### 2.3 Network Architecture & Flow Diagram

Data flow - sensor to database: The Arduino samples sensors continuously and prints formatted lines in serial to the Pi which reads each line in the autonomous loop thread (read_line()) to parse() converts it to a typed dict to _process_parsed() updates the shared _sensor_state dict under a threading lock to on each GPS message, if a valid moisture reading exists, a snapshot is written to SQLite via a background thread and the state['latest_reading'] cache is updated.

Data flow - operator command to motors: The operator clicks a button in control.html on their PC to a POST to http://localhost:8000/api/control to frontend.py proxies it to http://<Pi_IP>:5000/api/control to api/routes/control.py validates the command and calls the appropriate MotorController method to GPIO PWM signals drive the L298N to motors respond.

Data flow - live video: The Pi's camera stream thread encodes frames as JPEG at 10 FPS and holds the latest frame in _frame (under a lock) to /video_feed on the Pi serves a multipart MJPEG stream to frontend.py proxies the stream to control.html renders it in an <img> tag.

Network topology: All three tiers communicate over a local WiFi network. The Pi IP is configured once in frontend.py via the PI_SERVER environment variable (default http://10.***.***.118:5000). No internet connectivity is required during operation.

## 3. Implementation & Source Code

### 3.1 Repository Link

https://github.com/charlieM044/SIT210/tree/testbranch/dataflow/robo%20code%20final

### 3.2 Working Mechanism

the system is broken into 3 parts: The Arduino, raspberry pi, PC. The first part is the Arduino which handles the gps ultrasonic wall detection and moisture sensor which it then prints to the serial port. Each module has multiple different serial print dependent on the module reading which helps diagnose the problem. The pi controls the motors and camera and hosts a server backend to receive controls and send data. The PC hosts its own local api with a frontend html page that has the buttons that send the control commands to the pi.

## 4. Evaluation & Testing

### 4.1 Testing Approach

| Test no. | test name | Description: | Evaluation Criteria | method |
| --- | --- | --- | --- | --- |
| 1 | Wire Disconnects | What happens when a wire is disconnected from any component | Wire disconnection must not cause a critical system failure | Disconnect all wires one module at a time |
| 2 | Wall detection | Does the sensors currently detect walls | Does the ultra-sonic wall detection correctly detect walls and implement a proper turning solution | Leave robot to run in complicated area like under a dining table |
| 3 | Run time | Leave the robot running for an extended period. and check for any run time errors | If there are any errors or faulty data transfers occurring due to software. Things like memory leaks then fail | Leave the robot running for at least 30 mins without any critical errors occurring check terminal log on pi for final evaluation |
| 4. | Mode Switching | Switch between manual and autonomous mode while the robot is moving | Switching to manual stops motors immediately; switching to autonomous resumes the sensor loop; manual commands are correctly ignored in autonomous mode | Toggle mode repeatedly via the control panel while robot is in motion |
| 5 | Moisture Detection | Place the moisture sensor in dry, damp, and wet conditions | Readings are correctly classified as Minor/Moderate/Critical per the thresholds in config.py; Moderate and Critical readings trigger image capture | Submerge sensor tip to varying depths verify classification in the map and database |
| 6 | Motor controls | Drive the robot through an obstacle course | If the robot is unable to complete the course in a reasonable time without help then fail | Attempt to drive the robot through a maze and record thoughts of responsiveness and control |
| 7 | Picture | Does the frontend display the correct pictures | If the robot displays the incorrect images for each critical reading from the database, then pass | Compare the images being displayed to the time it was taken and ensure the image metadata matches the timestamp and gps data |

### 4.2 Evaluation Criteria

Each test is evaluated against a clear pass/fail condition:

Tests passed:

test 3: left the robot running for 1 hour with while periodically switching modes and manually switching

results: at the end of the wait the robot performed at expected and had no logged errors

test 5: the moisture sensor was repeatedly inserted and removed from a small bowl of water and the result was observed on the frontend monitor.

Result: after a max time of 5 seconds the live moisture feed updated with the correct reading after insertion and removal from the water

Test 4: the robot was repeatedly switched back and forth between modes

Results: the robot had no runtime errors and operate as expected.

Test 7: the robots database images were observed in the front end

Result: the robot images were confirmed to be confirmed to be correct

Tests inconclusive:

Test 2: the wall detection worked for a specific range that the ultra sonic sensors were rated to

Result: the angle and wall avoidance messages gave the correct response and the motors responded in the correct way. however, the due to cheap motor not turning properly and the limited range of the ultrasonic sensors the system lock the autonomous mode because any distance greater than 50cm will return as >1 leaving the ultrasonic sensor to fail the health check and disable the auto mode.

Tests failed:

Test 1: while monitoring the logs the robot had multiple sensors disconnected.

Results: each sensor displayed the correct sensor error message apart from the moisture sensor which would give floating pin numbers around 340 – 360 in the serial printout.

Corrective measure: include a check to health.py to check for and signs of floating pin using some kind of pattern recognition.

Test 6: attempt to navigate an obstacle course

Results: due to the cheap quality and improper mounting of the motors this fail catastrophically. One of the wheels shook its self off during the test and the turns left mush to be desired.

Corrective measure: install a second set of motors(preferably better quality ones) to the back and have them wired to the same power/ logic ENA controls on each side to preserve responsiveness and improve control capabilities.

Conclusion: most problems appear to stem from some kind of hardware/ real world design problems and not software and can be addressed with higher quality chasse design.

## 5. Advanced Design & Fault Tolerance (D/HD Requirements)

### 5.1 Fault Tolerance Strategy

The whole design is the modular so that if any one part fails the rest can continue to function rather than full crashing the system. That alongside the health check that the main pi runs to confirm the proper operation of each individual component and prevents any cascade failures due to a single component failing early in the loop.

Hardware abstraction and mock fallbacks: hardware/motors.py detects at import time whether it is running on a Raspberry Pi (config.IS_PI).  When On a non-Pi host it substitutes a MockGPIO class that prints pin state to stdout, allowing the full software stack to run and be tested on a PC without any hardware attached. Similarly, hardware/camera.py attempts three camera backends in order (picamera2 to picamera to OpenCV) and silently continues without a camera if all three fail.

Health monitoring with capability gates: hardware/health.py runs a background polling thread every 10 seconds that checks each subsystem (Arduino serial, camera, motors, storage, GPS, ultrasonic) independently for signs of failure. Each check sets a boolean flag (CAN_DRIVE, CAN_AVOID_WALLS, CAN_SAVE_DATA, CAN_CAPTURE, CAN_GPS). The can() function provides a single-line gate that autonomous.py can query before acting, preventing cascading failures.  For example, a camera failure will not prevent moisture data from being saved, and a GPS failure will not prevent images from being captured (they are just saved without coordinates).

Thread safety: The sensor state dict in autonomous.py, the ring buffer in saveData.py, and the camera frame buffer in camera.py, is protected by threading.Lock(). The Arduino serial port additionally uses a lock in arduino.py to prevent races between the autonomous loop, health checks, and the /api/arduino status endpoint reading the same serial buffer simultaneously.

Serial buffer management: After a Critical moisture event triggers a motor stop, clear_buffer() flushes the Arduino's serial input buffer. This prevents a backlog of stale messages (which may have accumulated during the stop) from being processed as new events when the robot starts again.

Persistent storage with in-memory cache: SQLite writes are performed in a daemon background thread so that a slow disk write never blocks the main sensor loop. The ring buffer (deque(maxlen=500)) is pre-warmed from the database on startup, so the frontend has immediate access to historical data even before new readings arrive. Data survives power loss as long as the background write thread completes before shutdown.

Graceful shutdown: main.py registers SIGINT and SIGTERM handlers that call motors.stop(), motors.cleanup(), and camera.release() before exiting, ensuring GPIO pins are released and no zombie PWM processes are left running.

### 5.2 Design Principles (HD Requirement)

Modularity and separation of concerns: The codebase is separated into three distinct layers with clearly defined layers. The hardware layer (hardware/) has all physical I/O and exposes simple method calls (motors.forward(), camera.capture_image(), read_line()). The API layer (api/) has HTTP routing and delegates to hardware and storage. The routes never touch GPIO directly. The data layer (saveData.py) owns persistence and exposes a storage interface that is independent of how data was collected. This means any layer can be replaced or tested in isolation; for example, the entire frontend can be developed and tested against a mock Pi by changing one environment variable.

Single responsibility: Each file has one job. state.py is purely shared runtime state. config.py is purely constants, no logic, just values, meaning hardware pin reassignments or threshold tuning require a change in exactly one file. autonomous.py owns the sensor loop and wall avoidance logic. health.py owns system monitoring. This reduces the surface area of bugs and makes the codebase navigable.

Singleton pattern for hardware resources: Each hardware driver (motors, camera, arduino, storage) is instantiated once at module level and imported as a singleton everywhere. This prevents multiple initialisations of the same GPIO pins or serial port, which would cause hardware conflicts, and ensures all parts of the system share a consistent view of hardware state (example: motors.current_speed).

Defensive input validation: saveData.py validates GPS coordinates and moisture values before writing (_validate()),thus rejecting bad values. api/routes/control.py validates commands with a whitelist before executing them. frontend.py validates mode strings and command names before proxying, so malformed requests are rejected.

Configuration over hardcoding: All changeable values like GPIO pins, serial port, camera resolution, ring buffer size and save interval are centralised in config.py. This satisfies the open/closed principle so the system can be adapted to different hardware (different pin wiring, different Pi models, different thresholds) without modifying any logic

## 6. User Manual

### 6.1 Installation Guide

Hardware prerequisites:

- Raspberry Pi 4 with Raspberry Pi OS (Bookworm or Bullseye)

- Arduino Nano 33 IoT connected via USB to the Pi

- L298N motor controller wired to BCM pins 17, 27, 22 (motor 1) and 23, 24, 25 (motor 2), check see config.py to change these. Then wire a 5v and ground

- Compatible camera module (picamera2, legacy picamera, or any OpenCV-compatible USB webcam)

- GPS module and moisture sensor wired to the Arduino per gps.ino and moisture.ino

- Two HC-SR04 ultrasonic sensors wired per ultrasonic.h

- Up to 12v of batteries wired to the 12v and ground to the L298N motor controller

Step 1: upload to the Arduino: Open Main.ino, gps.ino, moisture.ino, and ultrasonic.ino in the Arduino IDE. Install the Arduino_LSM6DS3 library via the Library Manager then upload. Confirm serial output in the Serial Monitor at 9600 baud.

Step 2: Set up the Pi:

Install the pi code to the pi.

bash

sudo apt update && sudo apt install python3-pip python3-venv -y

git clone < repo-url> && cd <repo-folder>

python3 -m venv venv && source venv/bin/activate

pip install flask flask-cors pyserial opencv-python RPi.GPIO picamera2

Step 3: Configure: Edit config.py if your GPIO wiring, serial port, or thresholds differ from the defaults. The serial port defaults to /dev/ttyACM0 on Pi and COM6 on Windows.

Step 4: Run the Pi server:

bash

source venv/bin/activate

python main.py

The server starts on 0.0.0.0:5000. Note the Pi's IP address (hostname -I).

Step 5: Set up the PC frontend:

bash

pip install flask flask-cors requests

PI_SERVER=http://<pi-ip>:5000 python frontend.py

Open http://localhost:8000 in your browser.

### 6.2 Operating Instructions

Starting the system: Power on the robot and ensure the Arduino is connected via USB before starting main.py. The Pi will try to start serial connection on boot, if the Arduino is not present a warning is printed but the system continues. You can configure your pi to start main.py on start up to remove the need for and manual work. To connect the front end and pi server each system must be on the same network for now.

Switching modes: The control panel at /control shows the current mode badge. Click MANUAL MODE to take direct control, or AUTONOMOUS to let the robot navigate and log data independently. In autonomous mode, manual movement commands are ignored (except for Stop which will always executes).

Manual driving: Use the on-screen directional pad or keyboard arrow keys to drive. Use the speed slider to set motor power (0-100%). Press Spacebar or the Stop button to halt immediately. The Emergency Stop button switches to manual mode and stops motors.

Monitoring readings: The /map page is supposed to displays all GPS tagged readings as colour coded markers (red = Critical >80%, amber = Moderate 60–80%, green = Minor <60%) on a live Leaflet map that refreshes every 15 seconds. Click any marker for details. The sidebar lists recent readings with severity and coordinates. (note due to time constraints the map page is depreciated)

Viewing the report: Navigate to /report for statistics, charts, and a full table of readings and images pulled from the database. You can print or export the data in different data forms, including csv, json, pdf.

## 7. Project Demonstration

- Video Link: [Insert Link to Recorded Demonstration Video]

## 8. Conclusion & Reflection

### 8.1 Summary

This project successfully demoed a semi-autonomous subfloor moisture inspection robot. The system integrates an Arduino Nano 33 IoT handling real-time sensor data (moisture, ultrasonic wall detection, GPS) with a Raspberry Pi 4 managing motor control, camera streaming, data persistence, and a Flask REST API backend. The PC-hosted frontend provides a live control panel, GPS map, and inspection report accessible from any browser on the local network. The robot can operate in autonomous mode navigating, detecting obstacles, and logging geo-referenced moisture readings with annotated images, or be switched to manual control at any time by the operator. All sensor data is stored persistently in SQLite and survives power cycles. The modular, layered architecture and hardware health monitoring ensure that the failure of any single component (camera, GPS, ultrasonic) degrades capability gracefully without crashing the rest of the system.

### 8.2 Reflection

This project demonstrated significant complexity that emerges when combining real-time hardware control with concurrent networking, and persistent data storage in a single embedded system. One of the most significant technical challenge was managing thread safety across the autonomous loop, Flask request handlers, health monitor, and camera stream. All this code running concurrently on the Pi caused many timing, memory and responsiveness issues. Early versions had many problems from serial buffer races where the health check and autonomous loop would consume each other's Arduino messages, producing erratic behaviour. This was resolved by adding a threading lock to all serial reads in arduino.py and calling clear_buffer() after critical events to remove old messages. Another challenge was the camera stream which caused constant crashes and memory leaks in the early stages of development, this was resolved by forcing the code to only open one instance of the camera at a time instead of 3 at init start up.

Many problems appeared to stem from my poor-quality craftsmanship rather than and kind of software. which makes sense as I had a limited budget and time to dedicate to physical chasses.

If the project were to be redone, the main changes that would be prioritised are, first, the wall avoidance algorithm which currently a simple stop-and-turn based on a distance threshold. this would be replaced with a more sophisticated sensor like lidar to ensure complete subfloor maping rather than random-walk exploration. second, using the Pi as a WiFi access point rather than relying on an existing network would make the system self-contained and deployable in locations without existing WiFi and remove the need for a manually configured host ip in frontend that changes constantly. And lastly low-quality motors and a makeshift chassis should be replaced with a higher quality purpose-built design rather than random parts found in the shed.

## 9. Bibliography

Reference list

Chubb Insurance Australia. (n.d.). Get Smart About Water Leaks | Chubb in Australia. [online] Chubb.com. Available at: https://www.chubb.com/au-en/individuals-families/campaign/water-damage.html [Accessed 1 Jun. 2026].

Héctor Azpúrua, Silva, Freitas, G., Clark, L., Ali–akbar Agha–mohammadi, Pessin, G., Campos, M. and Macharet, D.G. (2023). A Survey on the autonomous exploration of confined subterranean spaces: Perspectives from real-word and industrial robotic deployments. Robotics and Autonomous Systems, 160, pp.104304–104304. doi:https://doi.org/10.1016/j.robot.2022.104304.

Proximityplumbing.com.au. (2021). Causes, Timelines and Costs of Water Damage 2021. [online] Available at: https://proximityplumbing.com.au/blogs/causes-timelines-and-costs-of-water-damage-2021/.

TnTech-ECE (2025). GitHub - TnTech-ECE/Fall2022-Autonomous-Crawlspace-Inspection-Robot: Tennessee Tech ECE Capstone Project Autonomous Crawlspace Inspection Robot. [online] GitHub. Available at: https://github.com/TnTech-ECE/Fall2022-Autonomous-Crawlspace-Inspection-Robot [Accessed 1 Jun. 2026].

Wikipedia Contributors (2019). Roomba. [online] Wikipedia. Available at: https://en.wikipedia.org/wiki/Roomba.